EXPERIMENT 01

Aim

To simulate and compare the Bit Error Rate (BER) of Zigbee, 6LoWPAN, and NB-IoT technologies at different communication distances.

Short Procedure

- Import NumPy, Matplotlib, and SciPy libraries.

- Define the communication distance from 10 m to 1000 m.

- Set the transmission power, path loss, path loss exponent, and noise power for each technology.

- Calculate path loss, received power, and Signal-to-Noise Ratio (SNR).

- Calculate BER using the complementary error function (erfc).

- Plot BER against distance using a logarithmic scale for each technology.

PROGRAMINNG CODE

Import numpy as np

import matplotlib.pyplot as plt

from scipy.special import erfc

distance = np.linspace(10, 1000, 100)

# =====================================================

# Common BER equation

# BER = 0.5 * erfc(sqrt(SNR))

# =====================================================

# =====================================================

# 1. ZIGBEE

# =====================================================

Pt = 0          # Transmit power (dBm)

PL0 = 40        # Path loss at 1 m (dB)

n = 3.0         # Path loss exponent

N = -107  # Noise power (dBm)

PL = PL0 + 10*n*np.log10(distance)

Pr = Pt - PL

SNR = Pr - N

BER = 0.5 * erfc(np.sqrt(10**(SNR/10)))

print(BER[0])

plt.figure(figsize=(7,5))

plt.semilogy(distance, BER)

plt.xlabel("Distance (m)")

plt.ylabel("Bit Error Rate (BER)")

plt.title("Zigbee: BER vs Distance")

plt.grid(True)

plt.show()

# =====================================================

# 2. 6LoWPAN

# =====================================================

Pt = 0

PL0 = 40

n = 3.0

N = -107

PL = PL0 + 10*n*np.log10(distance)

Pr = Pt - PL

SNR = Pr - N

BER = 0.5 * erfc(np.sqrt(10**(SNR/10)))

print(BER[1])

plt.figure(figsize=(7,5))

plt.semilogy(distance, BER)

plt.xlabel("Distance (m)")

plt.ylabel("Bit Error Rate (BER)")

plt.title("6LoWPAN: BER vs Distance")

plt.grid(True)

plt.show()

# =====================================================

# 3. NB-IoT

# =====================================================

Pt = 23

PL0 = 31

n = 3.5

N = -107

PL = PL0 + 10*n*np.log10(distance)

Pr = Pt - PL

SNR = Pr - N

BER = 0.5 * erfc(np.sqrt(10**(SNR/10)))

print(BER[2])

plt.figure(figsize=(7,5))

plt.semilogy(distance, BER)

plt.xlabel("Distance (m)")

plt.ylabel("Bit Error Rate (BER)")

plt.title("NB-IoT: BER vs Distance")

plt.grid(True)

plt.show()

| N(dbm) | Zigbee(10) | 6LoWPAN(20) | NB-IoT(400) |
| --- | --- | --- | --- |
| 100 | 0.0 | 1.298 | 0.0 |
| 85 | 9.125 | 0.0024 | 1.0021 |
| 107 | 0.0 | 9.397 | 0.0 |

OUTPUT

Z=100

Z=85

Z=107

Result

The BER of Zigbee, 6LoWPAN, and NB-IoT was simulated and plotted successfully. The graphs illustrate how communication distance affects signal quality and bit error rate. Generally, as distance increases, SNR decreases and BER increases. NB-IoT uses different assumed transmission parameters, so its performance may differ from Zigbee and 6LoWPAN