EXPERIMENT 3

AIM

Benchmarking of 5G/6G Commumication for IoE Appliations based on Throughput, Latency, Reliability and Energy efficiency

Short work procedure

Import the Matplotlib library and define the number of IoE devices.

Set the assumed 5G network parameters, including capacity, latency, packet success rate, and energy consumption.

Calculate the offered traffic load and network utilization for each device count.

Calculate throughput, latency, reliability, and energy efficiency.

Display the calculated performance values for each device count.

Plot all four performance metrics in separate graphs using Matplotlib

Program code

import matplotlib.pyplot as plt

# Number of IoE devices

devices = [10, 50, 100, 200,267, 300,378, 400,467, 500,587,600]

# 5G network parameters (assumed)

capacity = 100                 # Mbps

base_latency = 10              # ms

packet_success_rate = 0.98

energy_per_packet = 0.001      # Joules

# IoE device parameters

packet_size = 1200             # Bytes

packets_per_second = 20

simulation_time = 10           # Seconds

# Store results

throughput = []

latency = []

reliability = []

energy_efficiency = []

for n in devices:

# Total packets generated

total_packets = n * packets_per_second * simulation_time

# Offered traffic load (Mbps)

offered_load = (

n * packets_per_second * packet_size * 8 / 1e6

)

# Network utilization

utilization = offered_load / capacity

# 1. Throughput (Mbps)

t = min(

offered_load * packet_success_rate,

capacity

)

# 2. Latency (ms): increases with congestion

u = min(utilization, 0.95)

l = base_latency + 10 * u / (1 - u)

# 3. Reliability (%): decreases with congestion

r = 100 * packet_success_rate * max(

0, 1 - 0.2 * utilization

)

# 4. Energy efficiency (Mbit/J)

successful_packets = total_packets * r / 100

total_energy = (

total_packets

* energy_per_packet

* (1 + utilization)

)

e = (

successful_packets * packet_size * 8

/ total_energy / 1e6

)

# Store values

throughput.append(t)

latency.append(l)

reliability.append(r)

energy_efficiency.append(e)

# Display results

print(f"\nIoE Devices: {n}")

print(f"Offered Load       : {offered_load:.2f} Mbps")

print(f"Throughput         : {t:.2f} Mbps")

print(f"Latency            : {l:.2f} ms")

print(f"Reliability        : {r:.2f} %")

print(f"Energy Efficiency  : {e:.2f} Mbit/J")

# Plot all four performance metrics

metrics = [

(throughput, "Throughput (Mbps)"),

(latency, "Latency (ms)"),

(reliability, "Reliability (%)"),

(energy_efficiency, "Energy Efficiency (Mbit/J)")

]

fig, axes = plt.subplots(2, 2, figsize=(11, 8))

for ax, (values, title) in zip(axes.flat, metrics):

ax.plot(devices, values, marker="o")

ax.set_title(title)

ax.set_xlabel("Number of IoE Devices")

ax.set_ylabel(title)

ax.grid(True)

plt.suptitle("5G Performance Benchmarking for IoE Applications")

plt.tight_layout()

plt.show()

Output

| IoE device | Offered load | Throughput (mbps) | Latency (ms) | Reliability (%) | Energy efficient |
| --- | --- | --- | --- | --- | --- |
| 267 | 51.26 | 50.24 | 20.52 | 87.95 | 5.58 |
| 378 | 72.88 | 71.12 | 36.46 | 83.78 | 4.66 |
| 467 | 89.66 | 87.87 | 96.75 | 80.43 | 4.07 |
| 587 | 112.70 | 10000 | 20000 | 75.91 | 3.43 |

Result

The 5G network performance for different numbers of IoE devices was simulated successfully. The graphs show how increasing device numbers affect throughput, latency, reliability, and energy efficiency. As the number of devices increases, network congestion and latency increase, while reliability and energy efficiency generally decrease. Throughput increases with traffic load until the network capacity limit is reached.

Program code 1

import matplotlib.pyplot as plt

# Number of IoE devices
devices = [10, 50, 100, 200, 300, 400, 500]

# 5G network parameters (assumed)
capacity = 100                 # Mbps
base_latency = 10              # ms
packet_success_rate = 0.98
energy_per_packet = 0.001      # Joules

# IoE device parameters
packet_size = 1200             # Bytes
packets_per_second = 20
simulation_time = 10           # Seconds

# Store results
throughput = []
latency = []
reliability = []
energy_efficiency = []

for n in devices:

    # Total packets generated
    total_packets = n * packets_per_second * simulation_time

    # Offered traffic load (Mbps)
    offered_load = (
        n * packets_per_second * packet_size * 8 / 1e6
    )

    # Network utilization
    utilization = offered_load / capacity

    # 1. Throughput (Mbps)
    t = min(
        offered_load * packet_success_rate,
        capacity
    )

    # 2. Latency (ms): increases with congestion
    u = min(utilization, 0.95)
    l = base_latency + 10 * u / (1 - u)

    # 3. Reliability (%): decreases with congestion
    r = 100 * packet_success_rate * max(
        0, 1 - 0.2 * utilization
    )

    # 4. Energy efficiency (Mbit/J)
    successful_packets = total_packets * r / 100

    total_energy = (
        total_packets
        * energy_per_packet
        * (1 + utilization)
    )

    e = (
        successful_packets * packet_size * 8
        / total_energy / 1e6
    )

    # Store values
    throughput.append(t)
    latency.append(l)
    reliability.append(r)
    energy_efficiency.append(e)

    # Display results
    print(f"\nIoE Devices: {n}")
    print(f"Offered Load       : {offered_load:.2f} Mbps")
    print(f"Throughput         : {t:.2f} Mbps")
    print(f"Latency            : {l:.2f} ms")
    print(f"Reliability        : {r:.2f} %")
    print(f"Energy Efficiency  : {e:.2f} Mbit/J")

# Plot all four performance metrics
metrics = [
    (throughput, "Throughput (Mbps)"),
    (latency, "Latency (ms)"),
    (reliability, "Reliability (%)"),
    (energy_efficiency, "Energy Efficiency (Mbit/J)")
]

fig, axes = plt.subplots(2, 2, figsize=(11, 8))

for ax, (values, title) in zip(axes.flat, metrics):
    ax.plot(devices, values, marker="o")
    ax.set_title(title)
    ax.set_xlabel("Number of IoE Devices")
    ax.set_ylabel(title)
    ax.grid(True)

plt.suptitle("5G Performance Benchmarking for IoE Applications")
plt.tight_layout()
plt.show()